## Definitions: Type of Construction

Frame

Buildings with exterior walls, floors, and roofs of combustible construction.

Masonry

Buildings with exterior walls of masonry or fire-resistive construction rated for not less than one hour and with combustible floors and roofs.

Non-Combustible

• buildings with exterior walls, floors, and roofs of noncombustible or slow-burning materials
• building supports of noncombustible or slow-burning materials
• noncombustible or slow-burning roof decks on noncombustible or slow-burning supports — regardless of the type of insulation on the roof surface

Masonry Non-Combustible

• buildings with exterior walls of masonry — not less than four inches thick, or
• buildings with exterior walls of fire-resistive construction — with a rating of not less than one hour, and
• noncombustible or slow-burning floors and roofs — regardless of the type of insulation on the roof surface

Modified Fire Resistive

• buildings with exterior walls, floors, and roofs of masonry materials described in the definition of fire resistive—less thick than required for fire-resistive structures but not less than four inches thick, or
• fire-resistive materials with a fire-resistance rating less than two hours but not less than one hour

Fire Resistive

• solid masonry, including reinforced concrete not less than four inches thick
• hollow masonry not less than 12 inches thick
• hollow masonry less than 12 inches thick, but not less than eight inches thick with a listed fire-resistance rating of not less than two hours
• assemblies with not less than a two-hour fire-resistance rating